Conformance Test Specification
Wi-Fi Throughput — Broadband Forum TR-398 Issue 3

DUT: Netgear Wi-Fi AP (802.11n)  |  STA: Linux System  |  Tools: iperf3 + PyATS + Wireshark

| Document ID | Standard | Version | Classification |
| --- | --- | --- | --- |
| CTS-TP-TR398-001 | Broadband Forum TR-398 Issue 3 (2022) | v1.0 | TCS CONFIDENTIAL |

1. Scope and Standard Reference

This Conformance Test Specification is derived from Broadband Forum Technical Report TR-398 Issue 3 (August 2022), "Wi-Fi In-Premises Performance Testing". TR-398 is the definitive industry standard for residential and enterprise Wi-Fi performance testing, used by ISPs, router OEMs, and certification labs worldwide.

Test case IDs, procedures, measurement methods, and pass/fail criteria are taken from TR-398 Issue 3 Sections 4.2 through 4.4. KPI values in Section 5 (Annex) Table A-1 define minimum performance requirements by device class. This implementation uses the "Wi-Fi 4 (802.11n) residential AP" device class.

TR-398 Issue 3 — Device Class: Wi-Fi 4 (802.11n) Residential AP DUT: Netgear 802.11n Access Point. Standard: TR-398 Issue 3 (2022). Applicable sections: §4.2 (Receiver Performance), §4.3 (Stability), §4.4 (Multiple Client). Not applicable: §4.5 (Wi-Fi 6 features), §4.6 (MLO). Test environment: iperf3 traffic generator + PyATS automation + Wireshark capture. KPI table: TR-398 Issue 3 Annex A, Table A-1, 802.11n column.

2. Test Environment

| Component | Role | Details |
| --- | --- | --- |
| Netgear Wi-Fi AP | DUT (Device Under Test) | 802.11n (Wi-Fi 4) ¦ 2.4/5 GHz ¦ Managed via SSH + Web UI |
| Linux System (Controller) | Test Automation | PyATS + pytest + iperf3 client + SSH/REST ¦ Wireshark capture |
| Linux System (STA) | Wi-Fi Station | Wi-Fi NIC ¦ iperf3 server ¦ Wavemon ¦ Wirelessly connected to DUT |
| iperf3 | Traffic Generator | TCP + UDP ¦ Multi-stream (-P 4) ¦ Bidirectional (--bidir) ¦ Duration-controlled |
| Wireshark/tshark | Packet Analyser | pcap per TC ¦ DSCP verification ¦ Loss analysis |
| PyATS | Framework | Cisco open-source network test automation ¦ Job execution ¦ Reporting |

3. Test Case Definitions — from TR-398 Issue 3

Each test case below is structured for AI extraction. The TCS GenAI pipeline reads: TC-ID (maps to TR-398 test ID), Standard Reference (exact section), Procedure (TR-398 prescribed method), and Pass Criterion (TR-398 Table A-1 KPI). These fields drive Test Case Generation (Step 1) and Test Script Generation (Step 2) separately.

3.1 TR-398 §4.2 — Receiver Performance Tests

Source: TR-398 Issue 3, Section 4.2 "Receiver Performance". These tests measure maximum throughput and connection capacity of the AP under controlled RF conditions.

| TC-ID | TR-398 Ref | Test Case Name | TR-398 Procedure (Summary) | Pass Criterion — KPI (TR-398 Table A-1) | Script: Test Case Step | Script: Test Script |
| --- | --- | --- | --- | --- | --- | --- |
| TC-TP-001 | TR-398 §4.2.1 | Maximum Connection Test | Establish maximum number of simultaneous TCP connections from STA to iperf3 server. Increment parallel streams until throughput degrades. | All connections established without session drops. Record maximum stable stream count. | TC generated: test_id=TC-TP-001, standard=TR-398-4.2.1, steps=[assoc, iperf3 -P 1→N, measure] | pytest: iperf3 -c <server> -P {N} -t 30 — assert 0 drops |
| TC-TP-002 | TR-398 §4.2.2 | Maximum Throughput | Measure maximum TCP downlink throughput. Run iperf3 for 60 seconds, 4 parallel streams, at close range (high SNR ≥ 40 dB). | TCP DL throughput — TR-398 Table A-1 802.11n: ≥ 150 Mbps @ 2.4GHz, ≥ 300 Mbps @ 5GHz | TC: test_id=TC-TP-002, kpi=tcp_dl_mbps_min, duration_s=60, streams=4 | pytest: iperf3 -c <server> -t 60 -P 4 -R — assert bitrate ≥ kpi |
| TC-TP-003 | TR-398 §4.2.3 | Maximum Uplink Throughput | Measure maximum TCP uplink throughput. Run iperf3 for 60 seconds, 4 parallel streams, STA to server direction. | TCP UL throughput — TR-398 Table A-1: ≥ 150 Mbps | TC: test_id=TC-TP-003, kpi=tcp_ul_mbps_min, duration_s=60, streams=4 | pytest: iperf3 -c <server> -t 60 -P 4 — assert bitrate ≥ kpi |
| TC-TP-004 | TR-398 §4.2.4 | Bidirectional Throughput | Measure simultaneous DL + UL throughput. Run iperf3 bidirectional for 60 seconds. Neither direction shall starve the other. | Combined DL+UL ≥ 200 Mbps. Each direction ≥ 80 Mbps independently. (TR-398 §4.2.4) | TC: test_id=TC-TP-004, kpi=[bidir_total, bidir_each], streams=4 | pytest: iperf3 --bidir -t 60 -P 4 — assert DL+UL, each direction |
| TC-TP-005 | TR-398 §4.2.5 | UDP Throughput and Latency | Measure UDP throughput at increasing offered loads (50, 100, 150 Mbps). Record maximum load with loss ≤ 1%. | UDP packet loss ≤ 1% at 100 Mbps offered load. (TR-398 §4.2.5) | TC: test_id=TC-TP-005, loads_mbps=[50,100,150], loss_pct_max=1 | pytest: iperf3 -u -b {load}M -t 30 — assert loss ≤ 1% |
| TC-TP-006 | TR-398 §4.2.6 | Latency Under Load | Measure ping RTT while iperf3 TCP download is active. Latency degradation under load is measured. | Ping RTT ≤ 50ms during active iperf3 DL. Idle RTT ≤ 10ms. (TR-398 §4.2.6) | TC: test_id=TC-TP-006, ping_idle_ms=10, ping_load_ms=50 | pytest: concurrent ping + iperf3 -t 60 — measure RTT both states |

3.2 TR-398 §4.3 — Stability Tests

Source: TR-398 Issue 3, Section 4.3 "Stability". These tests verify AP behaviour under repeated operations and sustained load. Results confirm long-term reliability for customer premises equipment.

| TC-ID | TR-398 Ref | Test Case Name | TR-398 Procedure (Summary) | Pass Criterion | Script Steps |
| --- | --- | --- | --- | --- | --- |
| TC-TP-007 | TR-398 §4.3.1 | Long Term Stability | Run iperf3 TCP DL for 10 minutes (600s) with 4 parallel streams. Monitor throughput per 30s interval. AP must not require restart. | Throughput variance ≤ 20% from baseline across all 30s intervals. 0 AP reboots. 0 unplanned disconnections. | TC→Script: iperf3 -t 600 -P 4 -i 30 ¦ assert stdev(intervals)/mean ≤ 0.20 |
| TC-TP-008 | TR-398 §4.3.2 | Association and Disassociation Stability | Perform 50 consecutive association / disassociation cycles. Each cycle: wpa_cli disconnect → reconnect → verify DHCP IP received. | 50/50 cycles complete. 0 AP reboots. 0 DHCP failures. Mean assoc time ≤ 15s per cycle. | TC→Script: loop 50× wpa_cli disconnect/reconnect — assert each cycle |

3.3 TR-398 §4.4 — Multiple Client Performance

Source: TR-398 Issue 3, Section 4.4 "Multiple Client Performance". These tests measure AP behaviour under concurrent multi-station load — the most operationally realistic scenario.

| TC-ID | TR-398 Ref | Test Case Name | TR-398 Procedure | Pass Criterion | Script Steps |
| --- | --- | --- | --- | --- | --- |
| TC-TP-009 | TR-398 §4.4.1 | Multi-Client Throughput | Associate 3 STAs simultaneously. Run iperf3 DL from AP to all 3 STAs concurrently. Measure per-STA and aggregate throughput. | Each STA achieves ≥ 40 Mbps DL. Aggregate throughput ≥ 120 Mbps. (TR-398 §4.4.1) | TC→Script: parallel iperf3 ×3 — measure per-STA + aggregate |
| TC-TP-010 | TR-398 §4.4.2 | Airtime Fairness | Under symmetric load from 3 STAs at equal distance, measure per-STA throughput. No STA shall be starved. | Per-STA throughput variance ≤ 20% from mean across all connected STAs. (TR-398 §4.4.2) | TC→Script: parallel iperf3 ×3 — assert stdev/mean ≤ 0.20 |

device: netgear_wifiap_80211n standard: 802.11n (Wi-Fi 4) tr398_class: Residential AP — 802.11n column kpis: # TR-398 Issue 3, Annex A, Table A-1 — 802.11n column tcp_dl_mbps_min: 150 # §4.2.2 Max Throughput tcp_ul_mbps_min: 150 # §4.2.3 Max Uplink bidir_total_mbps_min: 200 # §4.2.4 Bidirectional bidir_each_mbps_min: 80 # §4.2.4 each direction udp_loss_pct_max: 1 # §4.2.5 UDP Loss udp_load_mbps: 100 # §4.2.5 offered load ping_idle_ms_max: 10 # §4.2.6 Idle latency ping_load_ms_max: 50 # §4.2.6 Under load # TR-398 §4.3 Stability sustained_s: 600 # §4.3.1 10-minute run throughput_variance_pct: 20 # §4.3.1 stability KPI assoc_cycles: 50 # §4.3.2 cycle count assoc_time_s_max: 15 # §4.3.2 per cycle # TR-398 §4.4 Multi-Client multi_sta_count: 3 # §4.4.1 per_sta_mbps_min: 40 # §4.4.1 per-STA KPI airtime_variance_pct: 20 # §4.4.2 fairness not_applicable: - TR-398-4.5 # Wi-Fi 6 (802.11ax) — hardware not supported - TR-398-4.6 # MLO — 802.11be hardware required